CPE301 – SPRING 2019

# MIDTERM II

Student Name: Carlos D. Zuniga

Student #:2000783713

Student Email: zunigc2@unlv.nevada.edu

Primary Github address: https://github.com/cdaniel64/blassen

Directory: Midterms/Midterm2

- COMPONENTS LIST AND CONNECTION BLOCK DIAGRAM w/ PINS

List of Components used

Atmega328P

APDS9960

ESP-01

Breadboard

Power Supply

Jumper Cables

- INITIAL/MODIFIED/DEVELOPED CODE OF TASK 1/A

MAIN C Code

#define F_CPU 16000000UL		//initialize CPU

#include <avr/io.h>				//include avr library

#include <util/delay.h>			//include delay library

#include <stdio.h>				//include stdio library

#include "i2c_master.h"			//include i2c communication branch

#include "uart.h"				//include uart library branch

#include "apds.h"				//include apds library branch

#define BAUD 9600				//define transmission rate

#define BRGVAL (F_CPU/16/BAUD) - 1

#ifndef APDS_H

#define APDS_H

#include <avr/io.h>

#include "i2c_master.h"

#include "apds.h"

#define APDS_WRITE	(0x39 << 1) | 0

#define APDS_READ	(0x39 << 1) | 1

// Debug process as follows

// APDS-9960 I2C address

#define APDS9960_I2C_ADDR       0x39

//Error code for returned values

#define ERROR                   0xFF

//Acceptable device IDs

#define APDS9960_ID_1           0xAB

#define APDS9960_ID_2           0x9C

//Misc parameters

#define FIFO_PAUSE_TIME         30      // Wait period (ms) between FIFO reads

//APDS-9960 Reg address

#define APDS9960_ENABLE         0x80

#define APDS9960_ATIME          0x81

#define APDS9960_WTIME          0x83

#define APDS9960_PERS           0x8C

#define APDS9960_CONFIG1        0x8D

#define APDS9960_PPULSE         0x8E

#define APDS9960_CONFIG2        0x90

#define APDS9960_ID             0x92

#define APDS9960_RDATAL         0x96

#define APDS9960_RDATAH         0x97

#define APDS9960_GDATAL         0x98

#define APDS9960_GDATAH         0x99

#define APDS9960_BDATAL         0x9A

#define APDS9960_BDATAH         0x9B

#define APDS9960_POFFSET_UR     0x9D

#define APDS9960_POFFSET_DL     0x9E

#define APDS9960_CONFIG3        0x9F

//Bits

#define APDS9960_PON            0b00000001

#define APDS9960_AEN            0b00000010

#define APDS9960_PEN            0b00000100

#define APDS9960_WEN            0b00001000

#define APSD9960_AIEN           0b00010000

#define APDS9960_PIEN           0b00100000

#define APDS9960_GEN            0b01000000

#define APDS9960_GVALID         0b00000001

//On/Off definitions

#define OFF                     0

#define ON                      1

//Acceptable parameters for setMode

#define POWER                   0

#define AMBIENT_LIGHT           1

//#define PROXIMITY

#define WAIT                    3

#define AMBIENT_LIGHT_INT       4

#define ALL                     7

//LED Drive values

#define LED_DRIVE_100MA         0

#define LED_DRIVE_50MA          1

#define LED_DRIVE_25MA          2

#define LED_DRIVE_12_5MA        3

//LED Boost values

#define LED_BOOST_100           0

#define LED_BOOST_150           1

#define LED_BOOST_200           2

#define LED_BOOST_300           3

//Default values

#define DEFAULT_ATIME           219				// 103ms

#define DEFAULT_WTIME           246				// 27ms

#define DEFAULT_PROX_PPULSE     0x87			// 16us, 8 pulses

#define DEFAULT_POFFSET_UR      0				// 0 offset

#define DEFAULT_POFFSET_DL      0				// 0 offset

#define DEFAULT_CONFIG1         0x60			// No 12x wait (WTIME)

#define DEFAULT_LDRIVE          LED_DRIVE_100MA

#define DEFAULT_PGAIN           PGAIN_4X

#define DEFAULT_AGAIN           AGAIN_4X

#define DEFAULT_AILT            0xFFFF			// Force interrupt for calibration

#define DEFAULT_AIHT            0

#define DEFAULT_PERS            0x11			// 2 consecutive prox or ALS for int.

#define DEFAULT_CONFIG2         0x01			// No saturation interrupts or LED boost

#define DEFAULT_CONFIG3         0				// Enable all photodiodes, no SAI

#define DEFAULT_GLDRIVE         LED_DRIVE_100MA

#define DEFAULT_GWTIME          GWTIME_2_8MS

void apds_init();

void readColor();

#endif

FILE str_uart = FDEV_SETUP_STREAM(uart_putchar, NULL , _FDEV_SETUP_WRITE);

char results[256];

void init_UART();

int uart_putchar( char c, FILE *stream);

int main(void){

uint16_t red = 0, green = 0, blue = 0;

i2c_init();

init_UART();

stdout = &str_uart;

apds_init();

_delay_ms(2000);		//huge delays

printf("AT\r\n");

_delay_ms(5000);		//huge delays

printf("AT+CWMODE=1\r\n");

_delay_ms(5000);		//huge delays

printf("AT+CWJAP=\"Wifi\",\"Password\"\r\n");

while (1) {

_delay_ms(5000);	//huge delays so we dont get data loss

printf("AT+CIPMUX=0\r\n");

_delay_ms(5000);	//more huge delays

printf("AT+CIPSTART=\"TCP\",\"api.thingspeak.com\",80\r\n");

//setting up teamspeak

_delay_ms(5000);

readColor(&red, &green, &blue);

printf("AT+CIPSEND=104\r\n");

//preparing to connect to thingspeak

printf("GET https://api.thingspeak.com/update?api_key=TX0EL61BIR6SH2GO&field1=%05u&field2=%05u&field3=%05u\r\n", red, green, blue);

_delay_ms(5000);

}

}

void apds_init(){

uint8_t setup;

i2c_readReg(APDS_WRITE, APDS9960_ID, &setup,1);

if(setup != APDS9960_ID_1) while(1);

setup = 1 << 1 | 1<<0 | 1<<3 | 1<<4;

i2c_writeReg(APDS_WRITE, APDS9960_ENABLE, &setup, 1);

setup = DEFAULT_ATIME;

i2c_writeReg(APDS_WRITE, APDS9960_ATIME, &setup, 1);

setup = DEFAULT_WTIME;

i2c_writeReg(APDS_WRITE, APDS9960_WTIME, &setup, 1);

setup = DEFAULT_PROX_PPULSE;

i2c_writeReg(APDS_WRITE, APDS9960_PPULSE, &setup, 1);

setup = DEFAULT_POFFSET_UR;

i2c_writeReg(APDS_WRITE, APDS9960_POFFSET_UR, &setup, 1);

setup = DEFAULT_POFFSET_DL;

i2c_writeReg(APDS_WRITE, APDS9960_POFFSET_DL, &setup, 1);

setup = DEFAULT_CONFIG1;

i2c_writeReg(APDS_WRITE, APDS9960_CONFIG1, &setup, 1);

setup = DEFAULT_PERS;

i2c_writeReg(APDS_WRITE, APDS9960_PERS, &setup, 1);

setup = DEFAULT_CONFIG2;

i2c_writeReg(APDS_WRITE, APDS9960_CONFIG2, &setup, 1);

setup = DEFAULT_CONFIG3;

i2c_writeReg(APDS_WRITE, APDS9960_CONFIG3, &setup, 1);

}

void readColor(uint16_t *red, uint16_t *green, uint16_t *blue){

uint8_t redl, redh;

uint8_t greenl, greenh;

uint8_t bluel, blueh;

i2c_readReg(APDS_WRITE, APDS9960_RDATAL, &redl, 1);

i2c_readReg(APDS_WRITE, APDS9960_RDATAH, &redh, 1);

i2c_readReg(APDS_WRITE, APDS9960_GDATAL, &greenl, 1);

i2c_readReg(APDS_WRITE, APDS9960_GDATAH, &greenh, 1);

i2c_readReg(APDS_WRITE, APDS9960_BDATAL, &bluel, 1);

i2c_readReg(APDS_WRITE, APDS9960_BDATAH, &blueh, 1);

*red = redh << 8 | redl;

*green = greenh << 8 | greenl;

*blue = blueh << 8 | bluel;

}

void init_UART(void){

//Set baud rate

uint16_t baud_rate = BRGVAL;

UBRR0H = baud_rate >> 8;

UBRR0L = baud_rate & 0xFF;

//Enable receiver and transmitter

UCSR0B = ( 1 <<RXEN0)|( 1 <<TXEN0);

// Set frame format: 8data, 1stop bit

UCSR0C = (3 <<UCSZ00);

}

int uart_putchar(char c, FILE *stream){

//wait until buffer empty

while ( !( UCSR0A & ( 1 <<UDRE0)) );

//Put data into buffer

UDR0 = c;

return 0;

}

I2C CODE

#ifndef  F_CPU				//initialization of CPU

#define F_CPU 16000000UL

#endif

#include <avr/io.h>			//addition of common libraries

#include <util/twi.h>

#include "i2c_master.h"		//callout library of i2c master

#define F_SCL 100000UL		// SCL frequency

#define Prescaler 1

#define TWBR_val ((((F_CPU / F_SCL) / Prescaler) - 16 ) / 2)

void i2c_init(void)

{

TWBR = (uint8_t)TWBR_val;

}

uint8_t i2c_start(uint8_t address)

{

// reset TWI control register

TWCR = 0;

// transmit START condition

TWCR = (1<<TWINT) | (1<<TWSTA) | (1<<TWEN);

// wait for end of transmission

while( !(TWCR & (1<<TWINT)) );

// check if the start condition was transmitted

if((TWSR & 0xF8) != TW_START){ return 1; }

// load slave address into data register

TWDR = address;

// start transmission of address

TWCR = (1<<TWINT) | (1<<TWEN);

// wait for end of transmission

while( !(TWCR & (1<<TWINT)) );

// check if the device has READ / WRITE mode

uint8_t twst = TW_STATUS & 0xF8;

if ( (twst != TW_MT_SLA_ACK) && (twst != TW_MR_SLA_ACK) ) return 1;

return 0;

}

uint8_t i2c_write(uint8_t data)

{

// load data into data register

TWDR = data;

// start transmission of data

TWCR = (1<<TWINT) | (1<<TWEN);

// wait for end of transmission

while( !(TWCR & (1<<TWINT)) );

if( (TWSR & 0xF8) != TW_MT_DATA_ACK ){ return 1; }

return 0;

}

uint8_t i2c_read_ack(void)

{

// start TWI module and acknowledge data @ reception

TWCR = (1<<TWINT) | (1<<TWEN) | (1<<TWEA);

// wait for end of transmission

while( !(TWCR & (1<<TWINT)) );

// return received data from TWDR

return TWDR;

}

uint8_t i2c_read_nack(void)

{

// start receiving without acknowledging reception

TWCR = (1<<TWINT) | (1<<TWEN);

// wait for end of transmission

while( !(TWCR & (1<<TWINT)) );

return TWDR;

}

uint8_t i2c_transmit(uint8_t address, uint8_t* data, uint16_t length)

{

if (i2c_start(address | I2C_WRITE)) return 1;

for (uint16_t i = 0; i < length; i++)

{

if (i2c_write(data[i])) return 1;

}

i2c_stop();

return 0;

}

uint8_t i2c_receive(uint8_t address, uint8_t* data, uint16_t length)

{

if (i2c_start(address | I2C_READ)) return 1;

for (uint16_t i = 0; i < (length-1); i++)

{

data[i] = i2c_read_ack();

}

data[(length-1)] = i2c_read_nack();

i2c_stop();

return 0;

}

uint8_t i2c_writeReg(uint8_t devaddr, uint8_t regaddr, uint8_t* data, uint16_t length)

{

if (i2c_start(devaddr | 0x00)) return 1;

i2c_write(regaddr);

for (uint16_t i = 0; i < length; i++)

{

if (i2c_write(data[i])) return 1;

}

i2c_stop();

return 0;

}

uint8_t i2c_readReg(uint8_t devaddr, uint8_t regaddr, uint8_t* data, uint16_t length)

{

if (i2c_start(devaddr)) return 1;

i2c_write(regaddr);

if (i2c_start(devaddr | 0x01)) return 1;

for (uint16_t i = 0; i < (length-1); i++)

{

data[i] = i2c_read_ack();

}

data[(length-1)] = i2c_read_nack();

i2c_stop();

return 0;

}

void i2c_stop(void)

{

// transmit STOP condition when it is met

TWCR = (1<<TWINT) | (1<<TWEN) | (1<<TWSTO);

}

UART Code

#include <stdio.h>			//initialize library

#include <avr/io.h>

#include <avr/interrupt.h>

#include "uart.h"			//h.file

void init_UART(void){

//Set baud rate

uint16_t baud_rate = BRGVAL;

UBRR0H = baud_rate >> 8;

UBRR0L = baud_rate & 0xFF;

//Enable receiver and transmitter

UCSR0B = ( 1 <<RXEN0)|( 1 <<TXEN0);

// Set frame format: 8data, 1stop bit

UCSR0C = (3 <<UCSZ00);

}

int uart_putchar(char c, FILE *stream){

//wait until buffer empty

while ( !( UCSR0A & ( 1 <<UDRE0)) );

//Put data into buffer

UDR0 = c;

return 0;

}

APDS code

#include <avr/io.h>

#include "i2c_master.h"

#include "apds.h"

//initialization and processing of light sensing capabilities

void apds_init(){

uint8_t setup;

i2c_readReg(APDS_WRITE, APDS9960_ID, &setup,1);

if(setup != APDS9960_ID_1) while(1);

setup = 1 << 1 | 1<<0 | 1<<3 | 1<<4;

i2c_writeReg(APDS_WRITE, APDS9960_ENABLE, &setup, 1);

setup = DEFAULT_ATIME;

i2c_writeReg(APDS_WRITE, APDS9960_ATIME, &setup, 1);

setup = DEFAULT_WTIME;

i2c_writeReg(APDS_WRITE, APDS9960_WTIME, &setup, 1);

setup = DEFAULT_PROX_PPULSE;

i2c_writeReg(APDS_WRITE, APDS9960_PPULSE, &setup, 1);

setup = DEFAULT_POFFSET_UR;

i2c_writeReg(APDS_WRITE, APDS9960_POFFSET_UR, &setup, 1);

setup = DEFAULT_POFFSET_DL;

i2c_writeReg(APDS_WRITE, APDS9960_POFFSET_DL, &setup, 1);

setup = DEFAULT_CONFIG1;

i2c_writeReg(APDS_WRITE, APDS9960_CONFIG1, &setup, 1);

setup = DEFAULT_PERS;

i2c_writeReg(APDS_WRITE, APDS9960_PERS, &setup, 1);

setup = DEFAULT_CONFIG2;

i2c_writeReg(APDS_WRITE, APDS9960_CONFIG2, &setup, 1);

setup = DEFAULT_CONFIG3;

i2c_writeReg(APDS_WRITE, APDS9960_CONFIG3, &setup, 1);

}

void readColor(uint16_t *red, uint16_t *green, uint16_t *blue){

uint8_t redl, redh;

uint8_t greenl, greenh;

uint8_t bluel, blueh;

i2c_readReg(APDS_WRITE, APDS9960_RDATAL, &redl, 1);

i2c_readReg(APDS_WRITE, APDS9960_RDATAH, &redh, 1);

i2c_readReg(APDS_WRITE, APDS9960_GDATAL, &greenl, 1);

i2c_readReg(APDS_WRITE, APDS9960_GDATAH, &greenh, 1);

i2c_readReg(APDS_WRITE, APDS9960_BDATAL, &bluel, 1);

i2c_readReg(APDS_WRITE, APDS9960_BDATAH, &blueh, 1);

*red = redh << 8 | redl;

*green = greenh << 8 | greenl;

*blue = blueh << 8 | bluel;

}

- SCHEMATICS

- SCREENSHOTS OF EACH TASK OUTPUT (ATMEL STUDIO OUTPUT)

- SCREENSHOT OF EACH DEMO (BOARD SETUP)

- VIDEO LINKS OF EACH DEMO

https://youtu.be/Vc6xjqobPPA

- GITHUB LINK OF THIS DA

https://github.com/cdaniel64/blassen

Student Academic Misconduct Policy

http://studentconduct.unlv.edu/misconduct/policy.html

“This assignment submission is my own, original work”.